Platform and unmanned vehicle data exchange protocol

# Preface

This protocol adopts the MQTT publish/subscribe model for data exchange. Each vehicle terminal operates as an MQTT client, with the platform acting as the MQTT server. The relationship between the terminal and the platform is detailed below.

mqtt client

subscribe			      subscribe

mqtt server

mqtt client

publish				        publish

# MQTT Client Configuration

- User: (specified by the user)

- Password: (specified by the user)

- Client ID: Vehicle VIN (unique identifier)

- Connection URL: tcp://(server IP):3525

- Connection Timeout: Default is 10 seconds

- Heartbeat interval: 5s by default

- *.*.*.*:3525 indicates the server ip port, which changes according to the deployment environment. The default heartbeat interval is 5s (can be adjusted according to the actual situation). The connection timeout duration is 10 seconds by default (the value can be adjusted as required).

- Data format: application/json

- Object field definition specification: hump writing

# Real Time Operation Data Exchange

2.1. Real-Time Vehicle Data (Qos0):

Direction: Vehicle to Platform (periodic)

Frequency: 0.2Hz (adjustable) - once every 5 seconds

Topic: /vehicle/info/{vin}, where {vin} is the unique ID for the vehicle or transport means.

Vehicle Data Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Data Upload Timestamp | Long | Milliseconds |
| vehicleData | Vehicle Information | Object | Contains vehicle-specific data as defined in the vehicleData object below. |

VehicleData Object Definition:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| heading | Course Angle | Double | 0 is true north. Positive values clockwise, negative values anti-clockwise. Range: -180 to 180 degrees. |
| lon | Longitude | Double | East longitude is positive, west longitude is negative. |
| lat | Latitude | Double | North latitude is positive, south latitude is negative. |
| electric | Battery Life | Int | Estimated battery or fuel tank range, in km. |
| drivingMode | Driving Mode | Int | 0 for Autopilot, 1 for Manual Driving, 2 for Emergency Stop. |
| status | Vehicle Status | Int | 0 for Control Failure, 1 for Effective Control, 2 for Obstacles Ahead. |
| speed | Speed | Int | Vehicle speed in cm/s. |
| shiftPosition | Gear Position | Int | 0: P Gear, 1: R Gear, 2: N Gear, 3: D Gear. |
| doorStatus | Door Status | Int | 0: Open, 1: Closed. |

Example of Data Format:

jsonCopy code

{ "timestamp":1600050933000, "vehiceData":{ "heading": 0.0, "lon": 116.38846226084912, "lat": 39.99471532230262, "electric": "180", "drivingMode": 0, "acce": “10” "status": 1, "speed": 2000, "shiftPosition": 0, "doorStatus": 1 } }

2.2. Platform Response Feedback:

The platform does not provide an ACK response to the vehicle.

## Autonomous Driving Tasks and Vehicle Control:

3.1 Route/Path Task Delivery (Qos2):

Direction: Platform to Vehicle (On-demand interaction)

Topic: /cloud/driving/path/{vin}

Path Task Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task Identifier | String | UUID. |
| businessType | Business Type | Int | Varies: Loading, Distribution, Return, etc. |
| stopPointList | Point List | List | List of key points/locations along the route/path. |

StopPoint Definition:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| lon | Longitude | Double | Keep to 8 decimal places. |
| lat | Latitude | Double | Keep to 8 decimal places. |
| nodeID | Node ID | String | A string of five digits. |

Note:

The starting point is the current location/point of the vehicle, the last point in the route point list represents the end point of the task/route, and other points must be the points long the routes. The list does not represent the actual path, but only provides the key points/locations of the path for the vehicle to plan the path according to the vehicle map. In general, this list is sufficient to determine the vehicle routing result as the specified unique path.

Regardless of the nature of the vehicle task, the task normally ends when the vehicle reaches the last end point of the path and stops at the last point. The intermediate point is only used for path planning and the robot does not stop  there.

When performing non-previously defined tasks: the vehicle starts immediately after receiving the task;

When performing the defined task: the vehicle stops at the current position, knows that the robot enters the predetermined area (within a certain distance along the path), the vehicle starts, and performs the defiend operation. In principle, for defined tasks, the speed of the vehicle is automatically  adjusted according to the task specifics.

The end of a task : normal arrival at the end point, forced end via Cloud; Stop immediately when encountering obstacles; Stop immediately in case of system failure.

This task automatically ends when a fault or obstacle occurs. The vehicle makes its own decision if reverse driving is needed.

3.2 On-board Task Receiving Response Feedback (QoS2):

Direction: Platform to Vehicle

Topic: /vehicle/taskReceive/return/{vin}

Task Receiving Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task ID | String | UUID, matching the received task packet for task confirmation. |
| returnCode | Task Receive Result | Int | Receiving status codes for various conditions. 1. Normal reception 2. The path cannot be planned along the path points 3. Vehicle failure, unable to drive 4. Obstacles ahead, unable to drive |

3.3 Task Execution Feedback (QoS2):

Direction: Platform to Vehicle

Topic: /vehicle/taskFeedback/return/{{vin}

Task Feedback Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task Identifier | String | UUID, which is the same as the received task packet, is used by the cloud center to confirm the result of task executed, associated with the task ID |
| businessType | Business Type | Int | 1- Go to the loading point 2- Loading point to distribution point 3- Distribution point Return to loading point 4- Return to the loading point during delivery (the task is forced to terminate by the cloud or obstacles ahead) 5- Other |
| returnCode | Task Execution Result | Int | Execution status: 1. Complete normally and stop at the last point 2. The task is forced to end by the cloud and the vehicle stops 3. Vehicle failure, unable to drive 4. Obstacles ahead, unable to drive |
| lon | Longitude | Double | Longitude at the task's end point, positive for east, negative for west. |
| lat | Latitude | Double | Latitude at the task's end point, positive for north, negative for south. |

3.4 Cloud Task Termination Command (Qos2):

Direction: Platform to Vehicle (On-demand interaction)

Topic: /cloud/driving/cancel/{vin}

Task Termination Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task Identifier | String | UUID of the aborted task. |

Upon receiving this command, the vehicle immediately executes a stop and aborts the task.

3.5 Setting Door Control in the Cloud:

Direction: Platform to Vehicle (On-demand interaction)

Topic: /cloud/control/door/{vin}

Door Control Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task Identifier | String | UUID. |
| doorControl | Hatch Control | Int | 0 for opening the cargo hold, 1 for getting door status. |

3.6 Vehicle Response on Door Status:

Direction: Platform to Vehicle (On-demand interaction)

Topic: /vehicle/taskControlback/return/{vin}

Door Status Feedback Parameters:

| Parameter ID | Parameter Name | Type | Description |
| --- | --- | --- | --- |
| timestamp | Timestamp | Long | Timestamp of the command issued by the cloud. |
| taskId | Task Identifier | String | The UUID is the same as the received task packet. The cloud center can confirm the execution result associated with the task ID |
| ReturnCode | Return execution result | Int | 0- Cargo open 1- Cargo closed |